UGANDA CHRISTIAN UNIVERSITY

DEPARTMENT OF COMPUTING AND TECHNOLOGY

INTRODUCTION TO NETWORKS (CCNA 1)

GROUP PRACTICAL LAB 1

DESIGN, BUILD AND TEST A

SMALL OFFICE NETWORK

Implementation Lab Manual (Cisco Packet Tracer)

Work in groups. Follow the steps, enter the configurations, verify every result and document your evidence.

# Contents

1	Lab Background

2	Lab Scenario

3	Main Lab Objective

4	Group Formation

5	Learning Outcomes

6	Lab Roadmap and Suggested Schedule

7	Minimum Lab Requirements

8	Design and Addressing Reference

9	PART A: Planning and Subnetting

10	PART B: Build the Physical Network

11	PART C: Configure the End Devices

12	PART D: Configure the Switches

13	PART E: Configure the Router

14	PART F: IPv6 Addressing (Dual Stack)

15	PART G: Services and Protocol Analysis

16	PART H: Basic Network Security

17	PART I: Testing and Verification

18	Troubleshooting Challenge

19	Command Reference

20	Required Network Documentation

21	Group Discussion Questions

22	The Packet Journey Challenge

23	Presentation Structure

24	Required Deliverables

25	Assessment Rubric

26	Individual Participation

27	Extension Challenges

28	Expected Outcome

Appendix A: Reference Configurations

Appendix B: Group Submission Checklist

Appendix C: Common Mistakes and How to Reset a Device

# 1. Lab Background

Almost every office, school and clinic in Uganda now depends on a computer network: staff share printers, open files from a server, browse an intranet website and send e-mail. Behind that everyday experience is a small set of building blocks that you study in CCNA 1 (Introduction to Networks): end devices, switches, routers, cables, IP addresses, subnet masks, default gateways, MAC addresses, protocols and the layered models.

So far you have met these ideas in lectures. In this practical you will use them. Your group will act as a team of junior network technicians who must design, build, configure, test, secure and document a small office network in Cisco Packet Tracer. You will not be told only *what* to type. Every step explains why you type it, and you must prove the result with a verification command or a test.

By the end of the lab your group will have a working network of three LANs joined by one router, with a server that provides a web page and name resolution, and you will be able to explain how a message travels from one computer to another.

NOTE: This lab covers CCNA 1 practical skills only: cabling, IPv4 and IPv6 addressing, subnetting with VLSM, basic switch and router configuration, ARP, ICMP, TCP and UDP behaviour, DNS and HTTP, basic device security and troubleshooting. VLANs, trunking, OSPF and ACLs belong to CCNA 2 and CCNA 3 and are not needed here.

## 1.1 Topology for this lab

Figure 1 shows the network you will build. Study it carefully before you open Packet Tracer. Every device, interface and address you need appears in it. A high-resolution copy (CCNA1_Office_Topology_4K.png) is supplied with this manual so that you can print it or place it in your report.

Figure 1: Small office network topology (target design for this lab)

# 2. Lab Scenario

A new Kampala Small Office is being opened by the university. The ICT Directorate has asked your group to build the office network before the staff move in. The office has three working areas:

- Administration (LAN 1): up to 50 computers and devices in the long term. The area has two PCs and one shared network printer today.

- Operations (LAN 2): up to 25 devices. The area has two PCs and a laptop today.

- IT and Servers (LAN 3): up to 10 devices. It holds one server (intranet web server and name server) and two technician PCs.

The ICT Directorate has given you one block of private addresses: 192.168.10.0/24. You must divide it so that no addresses are wasted and there is room for growth. The office has one router, three switches and standard UTP cabling.

Your group must build a network that:

- Lets every device communicate with every other device in the office (same LAN and different LANs).

- Uses correct cables for every connection and correct IPv4 addressing (address, mask, gateway) for every device.

- Is managed securely: every switch and the router have passwords, a banner and SSH access, and unused switch ports are shut down.

- Provides a web page on the office server that users can open by name (http://intranet.office.local) instead of by IP address.

- Also works with IPv6 (dual stack) so that the office is ready for the future.

- Is documented well enough that another technician could rebuild it without asking you.

# 3. Main Lab Objective

Design, build, configure, verify, secure and document a small office network in Cisco Packet Tracer that demonstrates the practical CCNA 1 skills of cabling, addressing and subnetting, basic switch and router configuration, network testing and basic security, and explain how data moves through the network.

The final network should not be a collection of devices that merely show green lights. It should be a planned network:

Network layers used in this lab

End devices -> Access switches -> Router (default gateway) -> Other LANs and services

PCs,laptop     SW-ADMIN        R-OFFICE                  Server-IT:

printer        SW-OPS                                    web + DNS

server         SW-IT

# 4. Group Formation

Work in groups of four (three or five if your lecturer allows it). Every member takes one lead role per session and the roles rotate, so that everyone configures every kind of device. Even when you are not the lead, you must follow every step and be able to explain it. The lecturer may ask any member to explain or fix any part of the network.

| Role | Main responsibilities in the lab |
| --- | --- |
| Network designer and documenter | Subnet calculations, address tables, topology diagram, trouble tickets, report editing |
| Switch engineer | Switch configuration, port descriptions, port security, switch verification commands |
| Router engineer | Router configuration (IPv4 and IPv6), routing table checks, SSH on the router |
| Tester and troubleshooter | End-device configuration, test plan, evidence screenshots, fault diagnosis |

TIP: Use one shared folder (for example a cloud drive) with sub-folders pkt, configs, screenshots and report. Name Packet Tracer files with a version number: CCNA1_GroupX_v1.pkt, v2, v3. Never overwrite the previous version. If Packet Tracer crashes you only lose the last step.

# 5. Learning Outcomes

After completing this lab, each student can:

| No. | Outcome | Where practised |
| --- | --- | --- |
| LO1 | Identify network devices and choose the correct cable type for each connection | Part B |
| LO2 | Calculate IPv4 subnets with VLSM and assign network, host, gateway and broadcast addresses | Part A |
| LO3 | Configure static IPv4 and IPv6 addresses on end devices and verify them with ipconfig | Parts C, F |
| LO4 | Perform basic configuration of a Cisco switch (names, passwords, banner, management address, descriptions) | Part D |
| LO5 | Perform basic configuration of a Cisco router (interfaces, addresses, passwords, banner) and read the routing table | Parts E, F |
| LO6 | Explain and observe ARP, ICMP, TCP, UDP, DNS and HTTP, and relate them to the OSI and TCP/IP models | Part G |
| LO7 | Secure devices with passwords, SSH, port security and by disabling unused ports | Part H |
| LO8 | Test connectivity with ping and tracert, and troubleshoot faults with a systematic method | Parts I and Section 18 |
| LO9 | Document a network (diagram, tables, configurations) and present it | Sections 20 to 24 |

# 6. Lab Roadmap and Suggested Schedule

| Part | Title | Main concepts | Marks |
| --- | --- | --- | --- |
| A | Planning and subnetting | Binary, subnet mask, VLSM, network/host/broadcast address | 15 |
| B | Build the physical network | Devices, UTP cables, link lights, interface naming | 10 |
| C | Configure the end devices | IP address, mask, default gateway, DNS, ipconfig | 10 |
| D | Configure the switches | IOS modes, hostname, passwords, banner, SVI, MAC table | 15 |
| E | Configure the router | Interfaces, descriptions, routing table, inter-LAN ping | 15 |
| F | IPv6 addressing | Global unicast, link-local, dual stack | 5 |
| G | Services and protocol analysis | DNS, HTTP, ARP, ICMP, TCP, UDP, OSI and TCP/IP in Simulation mode | 10 |
| H | Basic security | SSH, port security, unused ports | 5 |
| I | Testing, troubleshooting, documentation | Test plan, fault tickets, final documents | 5 + 10 |

Suggested plan (about three lab sessions of two hours, plus preparation time and a presentation session):

| Session | Focus | What to show the lecturer before leaving |
| --- | --- | --- |
| 1 | Parts A, B and C | Completed subnet worksheet; all links green; every end device addressed; ping inside each LAN works |
| 2 | Parts D, E and F | Switches and router configured and saved; ping between all three LANs works; IPv6 ping works |
| 3 | Parts G, H and I | DNS and web page working; SSH working; port security demonstrated; test plan filled; two faults fixed |
| 4 | Presentation | Live demonstration and questions |

# 7. Minimum Lab Requirements

## 7.1 Software

| Software | Version | Use |
| --- | --- | --- |
| Cisco Packet Tracer | 8.2 or later (free with a Cisco Networking Academy account) | Build, configure and test the network |
| Word processor and a screenshot tool | Any | Report and evidence |
| Calculator with binary and decimal modes (or the Windows Programmer calculator) | Any | Subnet calculations (you must also be able to calculate by hand) |

## 7.2 Devices and cables you must use

| Device | Packet Tracer model | Quantity | Purpose |
| --- | --- | --- | --- |
| Router | 2911 (three Gigabit interfaces) | 1 | R-OFFICE, the default gateway for all LANs |
| Switches | 2960-24TT | 3 | SW-ADMIN, SW-OPS, SW-IT (access layer) |
| PCs | PC-PT | 6 | PC-ADM1, PC-ADM2, PC-OPS1, PC-OPS2, PC-IT1, PC-IT2 |
| Laptop | Laptop-PT | 1 | Laptop-OPS (wired Ethernet in this lab) |
| Printer | Printer-PT | 1 | Printer-ADM (network printer with a static IP) |
| Server | Server-PT | 1 | Server-IT (web server and DNS server) |
| Cables | Copper Straight-Through | 12 | 9 end-device links to switches and 3 router-to-switch links |

NOTE: Total: 4 network devices (1 router and 3 switches) and 9 end devices (6 PCs, 1 laptop, 1 printer, 1 server). Do not use the automatic cable (the lightning-bolt icon). Choosing the cable yourself is part of the exercise. A wrong cable type is one of the troubleshooting faults.

## 7.3 Minimum counts for this lab

- Routers: 1 and switches: 3

- End devices: 9 (6 PCs, 1 laptop, 1 printer, 1 server)

- LANs: 3 (three IPv4 subnets and three IPv6 /64 prefixes)

- Cables: 12 (9 end-device links and 3 router-to-switch links)

- Services: 2 (DNS and HTTP on Server-IT)

# 8. Design and Addressing Reference

This section is the single source of truth for the lab. Work through Part A first and calculate your own answers. Then compare with this section. If your group deliberately changes a value, change it here first, tell every member, and update your documentation.

## 8.1 Requirements for the subnet calculation

| LAN | Department | Hosts needed (future) | Devices today | Notes |
| --- | --- | --- | --- | --- |
| LAN 1 | Administration | 50 | 2 PCs + 1 printer | Largest network. It will grow. |
| LAN 2 | Operations | 25 | 2 PCs + 1 laptop | Medium network |
| LAN 3 | IT and Servers | 10 | 1 server + 2 PCs | Smallest network |

Remember that every LAN also needs one address for the router interface (default gateway) and one for the switch management address, and that the network address and broadcast address cannot be given to a device. Allocate the largest network first (this method is called VLSM, Variable Length Subnet Masking) and start each new subnet immediately after the previous one.

## 8.2 Answers to check after you finish Part A

| LAN | Prefix and mask | Network address | First usable | Last usable | Broadcast | Usable hosts |
| --- | --- | --- | --- | --- | --- | --- |
| LAN 1 Admin | /26 255.255.255.192 | 192.168.10.0 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 | 62 |
| LAN 2 Operations | /27 255.255.255.224 | 192.168.10.64 | 192.168.10.65 | 192.168.10.94 | 192.168.10.95 | 30 |
| LAN 3 IT | /28 255.255.255.240 | 192.168.10.96 | 192.168.10.97 | 192.168.10.110 | 192.168.10.111 | 14 |
| Unused (growth) | 192.168.10.112 to 192.168.10.255 |  |  |  |  | Reserved for future LANs |

## 8.3 IPv4 addressing table

| Device | Interface | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| R-OFFICE | G0/0 (to SW-ADMIN) | 192.168.10.1 | 255.255.255.192 | n/a |
| R-OFFICE | G0/1 (to SW-OPS) | 192.168.10.65 | 255.255.255.224 | n/a |
| R-OFFICE | G0/2 (to SW-IT) | 192.168.10.97 | 255.255.255.240 | n/a |
| SW-ADMIN | VLAN 1 | 192.168.10.2 | 255.255.255.192 | 192.168.10.1 |
| SW-OPS | VLAN 1 | 192.168.10.66 | 255.255.255.224 | 192.168.10.65 |
| SW-IT | VLAN 1 | 192.168.10.98 | 255.255.255.240 | 192.168.10.97 |
| PC-ADM1 | NIC | 192.168.10.10 | 255.255.255.192 | 192.168.10.1 |
| PC-ADM2 | NIC | 192.168.10.11 | 255.255.255.192 | 192.168.10.1 |
| Printer-ADM | NIC | 192.168.10.20 | 255.255.255.192 | 192.168.10.1 |
| PC-OPS1 | NIC | 192.168.10.70 | 255.255.255.224 | 192.168.10.65 |
| PC-OPS2 | NIC | 192.168.10.71 | 255.255.255.224 | 192.168.10.65 |
| Laptop-OPS | NIC | 192.168.10.72 | 255.255.255.224 | 192.168.10.65 |
| Server-IT | NIC | 192.168.10.100 | 255.255.255.240 | 192.168.10.97 |
| PC-IT1 | NIC | 192.168.10.101 | 255.255.255.240 | 192.168.10.97 |
| PC-IT2 | NIC | 192.168.10.102 | 255.255.255.240 | 192.168.10.97 |

NOTE: DNS server for all end devices: 192.168.10.100 (Server-IT). Domain name of the office: office.local. Hosts are numbered so that the last octet tells you the department at a glance (10s = Admin, 70s = Operations, 100s = IT).

## 8.4 IPv6 addressing table (used in Part F)

| Device | Interface | IPv6 address / prefix | Link-local |
| --- | --- | --- | --- |
| R-OFFICE | G0/0 | 2001:DB8:ACAD:10::1/64 | FE80::1 |
| R-OFFICE | G0/1 | 2001:DB8:ACAD:20::1/64 | FE80::1 |
| R-OFFICE | G0/2 | 2001:DB8:ACAD:30::1/64 | FE80::1 |
| PC-ADM1 / PC-ADM2 | NIC | 2001:DB8:ACAD:10::10 and ::11 /64 | automatic |
| PC-OPS1 / PC-OPS2 / Laptop-OPS | NIC | 2001:DB8:ACAD:20::70, ::71 and ::72 /64 | automatic |
| Server-IT / PC-IT1 / PC-IT2 | NIC | 2001:DB8:ACAD:30::100, ::101 and ::102 /64 | automatic |

## 8.5 Switch and router port map

| Switch | Fa0/1 | Fa0/2 | Fa0/3 | Gi0/1 (uplink) | Fa0/4 to Fa0/24, Gi0/2 |
| --- | --- | --- | --- | --- | --- |
| SW-ADMIN | PC-ADM1 | PC-ADM2 | Printer-ADM | R-OFFICE G0/0 | Unused (shut down in Part H) |
| SW-OPS | PC-OPS1 | PC-OPS2 | Laptop-OPS | R-OFFICE G0/1 | Unused (shut down in Part H) |
| SW-IT | Server-IT | PC-IT1 | PC-IT2 | R-OFFICE G0/2 | Unused (shut down in Part H) |

## 8.6 Lab credentials (for the lab only)

| Item | Value used in this manual |
| --- | --- |
| Privileged EXEC (enable) secret | Ucu@Enable2026 |
| Console password | Ucu@Con2026 |
| VTY (Telnet) password, used until Part H | Ucu@Vty2026 |
| SSH local administrator (Part H) | netadmin / Ucu@Admin2026 |
| Domain name | office.local |

WARNING: These passwords are printed in a manual, so they are not secure in real life. In a real network every device gets unique, long passwords kept in a password manager. Your report must explain what a good password policy looks like.

# 9. PART A: Planning and Subnetting

Good network engineers plan on paper before they touch a device. In this part you decide how the single address block 192.168.10.0/24 is shared between the three LANs. You will use binary numbers and the subnet mask, which are the foundation of everything else in the lab.

## 9.1 Concepts you must understand first

### IPv4 address, subnet mask and prefix

An IPv4 address has 32 bits, written as four octets (for example 192.168.10.10). The subnet mask has the same shape. In the mask, every 1 marks a bit of the network part and every 0 marks a bit of the host part. The prefix length (for example /26) counts the 1s. Two devices are in the same network only if the network part of their addresses is identical.

| Prefix | Mask in binary (last octet) | Mask in decimal | Host bits | Addresses per subnet | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| /24 | 00000000 | 255.255.255.0 | 8 | 256 | 254 |
| /25 | 10000000 | 255.255.255.128 | 7 | 128 | 126 |
| /26 | 11000000 | 255.255.255.192 | 6 | 64 | 62 |
| /27 | 11100000 | 255.255.255.224 | 5 | 32 | 30 |
| /28 | 11110000 | 255.255.255.240 | 4 | 16 | 14 |
| /29 | 11111000 | 255.255.255.248 | 3 | 8 | 6 |
| /30 | 11111100 | 255.255.255.252 | 2 | 4 | 2 |

Formula: usable hosts = 2 to the power of h, minus 2, where h is the number of host bits. The two addresses removed are the network address (all host bits 0) and the broadcast address (all host bits 1).

### How a computer decides: local or remote? (the AND operation)

Before sending, a host performs a bitwise AND of its own address and mask, and of the destination address and the same mask. If the two results are equal, the destination is on the same network and the host sends the frame directly. If the results differ, the destination is remote, and the host sends the frame to its default gateway.

| Step | Binary (last octet only) | Result |
| --- | --- | --- |
| Source PC-ADM1 = 192.168.10.10 | 00001010 | Mask /26 = 11000000 |
| 10 AND mask | 00001010 AND 11000000 = 00000000 | Network 192.168.10.0 |
| Destination PC-OPS1 = 192.168.10.70 | 01000110 | Mask (PC-ADM1 uses) /26 = 11000000 |
| 70 AND mask | 01000110 AND 11000000 = 01000000 = 64 | Network 192.168.10.64 |
| Compare | 0 is not equal to 64 | Different network: send to the default gateway 192.168.10.1 |

TASK A1  Binary and decimal practice

Complete this table by hand first, then check with a calculator. Show your working in your notebook. These conversions are examination skills.

| Decimal octet | Binary |  | Binary | Decimal octet |
| --- | --- | --- | --- | --- |
| 10 |  |  | 11000000 |  |
| 70 |  |  | 11100000 |  |
| 100 |  |  | 11110000 |  |
| 192 |  |  | 01100100 |  |
| 224 |  |  | 00001010 |  |

TASK A2  VLSM subnet calculation

Use the requirements in Section 8.1. Follow this method for each LAN, starting with the largest:

- Write the number of hosts needed. Add the router interface and the switch management address if they are not already counted.

- Find the smallest h such that 2 to the power h minus 2 is at least the hosts needed. This gives the host bits and therefore the prefix: prefix = 32 minus h.

- Convert the prefix into a subnet mask.

- The network address is the first address of the block. The first block starts at 192.168.10.0. Each following block starts immediately after the broadcast address of the previous one.

- The first usable address is the network address plus 1. The broadcast is the network address plus the block size minus 1. The last usable is the broadcast minus 1.

| LAN | Hosts needed | Host bits h | Prefix | Mask | Network | First usable | Last usable | Broadcast | Usable hosts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LAN 1 Admin | 50 |  |  |  |  |  |  |  |  |
| LAN 2 Ops | 25 |  |  |  |  |  |  |  |  |
| LAN 3 IT | 10 |  |  |  |  |  |  |  |  |

Worked example for LAN 1. 50 hosts need h = 6, because 2 to the 5 is 32 minus 2 = 30 (too small) and 2 to the 6 is 64 minus 2 = 62 (large enough). The prefix is 32 minus 6 = /26, the mask is 255.255.255.192, the network is 192.168.10.0, the broadcast is 0 + 64 - 1 = 192.168.10.63, and the first and last usable are .1 and .62. The next block therefore starts at 192.168.10.64. Now do LAN 2 and LAN 3 yourselves, then compare with Section 8.2.

TASK A3  Explain your choices

Answer in writing (two or three sentences each). These answers go into your report.

- Why did we choose /26, /27 and /28 instead of using /24 on every LAN?

- What would happen if we gave LAN 3 (10 hosts) a /29?

- Which addresses in 192.168.10.0/24 are still free after the three subnets? What could the office use them for?

- Why must the router interface in each LAN be inside that LAN's subnet?

- Is 192.168.10.0 a valid host address for PC-ADM1? Is 192.168.10.63? Explain.

TASK A4  Sketch the topology and addressing on paper

Without looking at Figure 1, draw the network from the description in Section 2. Label every device, interface and address from your calculations. Compare your sketch with Figure 1 and note differences. Show the sketch and the completed A2 table to your lecturer before you start Part B (Checkpoint A).

CHECKPOINT: Checkpoint A: completed binary table, completed VLSM table with correct answers, written answers to A3, and a labelled sketch.

# 10. PART B: Build the Physical Network

You now create the network in Packet Tracer. In real life this is the work of installing equipment and running cables. Take care: a single wrong cable or interface causes a lot of confusion later.

## 10.1 Concepts: cables and ports

| Cable | Connects | Packet Tracer symbol | Use in this lab |
| --- | --- | --- | --- |
| Straight-through UTP | Different kinds of devices: PC to switch, router to switch, server to switch, printer to switch | Solid black line | All 12 links in this lab |
| Crossover UTP | Same kinds of devices: PC to PC, switch to switch, router to router, PC to router | Dashed black line | None (used only in the optional test in B4) |
| Console (rollover) | PC serial port (RS-232) to router or switch console port, for configuration | Light blue line | Optional. Used in Part D and E to open the CLI from a PC |
| Fibre / serial | Long distances / WAN links | Orange / red | Not used |

In Packet Tracer the link light at each end shows the state: green = the link is up; red = the link is down (wrong cable, disabled port or no power); amber on a switch = the port is still starting up (spanning tree), which becomes green after about 30 seconds. Router interfaces are shut down by default, so links to the router stay red until you configure them in Part E.

TASK B1  Place and name the devices

- Open Packet Tracer and create a new file. Save it immediately as CCNA1_GroupX_v1.pkt.

- Open Options > Preferences and tick Always Show Port Labels in Logical Workspace, so that every interface name appears on the diagram.

- From the device bar choose Network Devices > Routers and place one 2911. Choose Switches and place three 2960-24TT.

- Choose End Devices and place six PC, one Laptop, one Printer and one Server.

- Arrange them like Figure 1: the router at the top, three switches in a row below it, and the end devices under their switch.

- Rename every device by clicking its name label (display name). Use exactly the names in Section 8: R-OFFICE, SW-ADMIN, SW-OPS, SW-IT, PC-ADM1, PC-ADM2, Printer-ADM, PC-OPS1, PC-OPS2, Laptop-OPS, Server-IT, PC-IT1, PC-IT2.

TASK B2  Connect the devices

Choose Connections > Copper Straight-Through (the solid black line). Click the first device, select the interface, click the second device, select the interface. Use exactly the ports in Section 8.5.

| From device and port | To device and port | Cable | Name of this link |
| --- | --- | --- | --- |
| R-OFFICE G0/0 | SW-ADMIN Gi0/1 | Straight-through | Router uplink LAN 1 |
| R-OFFICE G0/1 | SW-OPS Gi0/1 | Straight-through | Router uplink LAN 2 |
| R-OFFICE G0/2 | SW-IT Gi0/1 | Straight-through | Router uplink LAN 3 |
| PC-ADM1 FastEthernet0 | SW-ADMIN Fa0/1 | Straight-through |  |
| PC-ADM2 FastEthernet0 | SW-ADMIN Fa0/2 | Straight-through |  |
| Printer-ADM FastEthernet0 | SW-ADMIN Fa0/3 | Straight-through |  |
| PC-OPS1 FastEthernet0 | SW-OPS Fa0/1 | Straight-through |  |
| PC-OPS2 FastEthernet0 | SW-OPS Fa0/2 | Straight-through |  |
| Laptop-OPS FastEthernet0 | SW-OPS Fa0/3 | Straight-through |  |
| Server-IT FastEthernet0 | SW-IT Fa0/1 | Straight-through |  |
| PC-IT1 FastEthernet0 | SW-IT Fa0/2 | Straight-through |  |
| PC-IT2 FastEthernet0 | SW-IT Fa0/3 | Straight-through |  |

TASK B3  Verify the physical layer

- Wait about 30 seconds. All switch-to-end-device links should turn green. The three router-to-switch links should show a red end at the router and a green or red end at the switch. This is correct, because the router interfaces are still shut down.

- If an end-device link is red, check: wrong port, wrong cable type, or the device is powered off (click the power switch on the device, in the Physical tab).

- Take a screenshot of the whole topology with port labels visible (evidence B3).

NOTE: Packet Tracer has a Realtime and a Simulation mode, shown at the bottom right. The fast-forward button next to the clock speeds up time so that the ports turn green sooner.

TASK B4  Optional experiment: the wrong cable

Temporarily connect PC-ADM1 directly to PC-ADM2 with a straight-through cable. The lights stay red or amber and the PCs cannot communicate on older models. Replace with a crossover cable and observe the difference. Explain in two sentences why PC-to-PC needs a crossover cable but PC-to-switch does not (hint: transmit and receive pairs, and Auto-MDIX on modern ports). Restore the original links before continuing.

CHECKPOINT: Checkpoint B: topology screenshot with port labels, all end-device links green, answers to B4.

# 11. PART C: Configure the End Devices

A computer needs four values to communicate beyond its own network: an IP address, a subnet mask, a default gateway and (for names) a DNS server. In this part you set these values manually (static addressing) on every end device, using the table in Section 8.3.

## 11.1 Concepts

| Value | Purpose | What happens if it is wrong |
| --- | --- | --- |
| IP address | Identifies the device (Layer 3) in its network | Duplicate address: unreliable connection. Address in the wrong subnet: cannot reach the gateway |
| Subnet mask | Tells the device which part of the address is the network | Wrong mask: device misjudges which hosts are local and which are remote |
| Default gateway | The router interface used to leave the local network | Local ping works, but every remote destination fails |
| DNS server | Converts names (intranet.office.local) to IP addresses | Ping by IP works, but names do not resolve |

TASK C1  Configure the PCs and the laptop

- Click PC-ADM1. Select the Desktop tab, then IP Configuration.

- Select Static. Enter the IPv4 address, subnet mask, default gateway and DNS server from Section 8.3 (for PC-ADM1: 192.168.10.10, 255.255.255.192, 192.168.10.1, DNS 192.168.10.100).

- Close the window. The values are saved automatically.

- Repeat for PC-ADM2, PC-OPS1, PC-OPS2, Laptop-OPS, PC-IT1 and PC-IT2. Be careful with the mask: the three LANs use three different masks.

TASK C2  Configure the printer and the server

- Click Printer-ADM. Open the Config tab, then FastEthernet0. Choose Static and enter 192.168.10.20 / 255.255.255.192. Then open Settings (Global) and enter the Gateway 192.168.10.1 and DNS 192.168.10.100.

- Click Server-IT. Open Config > FastEthernet0, choose Static and enter 192.168.10.100 / 255.255.255.240. In Settings (Global) enter the Gateway 192.168.10.97.

TASK C3  Verify the configuration of every end device

On each PC open the Command Prompt (Desktop tab) and run:

PC Command Prompt

C:\> ipconfig

C:\> ipconfig /all

Check that the address, mask, gateway and DNS agree with Section 8.3. Note the physical (MAC) address of each device in a table. You will use the MAC addresses in Part D. Create this table in your report:

| Device | MAC address (ipconfig /all) | IPv4 address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC-ADM1 |  |  |  |  |
| PC-ADM2 |  |  |  |  |
| PC-OPS1 |  |  |  |  |
| PC-IT1 |  |  |  |  |

TASK C4  Test connectivity before the router is configured

The switches work without configuration (they learn MAC addresses by themselves), but the router interfaces are still shut down. Predict the result of each test before you run it, then run it with ping from the Command Prompt.

| Test | Prediction (works / fails) | Actual result | Explanation |
| --- | --- | --- | --- |
| PC-ADM1 to PC-ADM2 (192.168.10.11) |  |  |  |
| PC-ADM1 to Printer-ADM (192.168.10.20) |  |  |  |
| PC-OPS1 to Laptop-OPS (192.168.10.72) |  |  |  |
| PC-IT1 to Server-IT (192.168.10.100) |  |  |  |
| PC-ADM1 to its gateway (192.168.10.1) |  |  |  |
| PC-ADM1 to PC-OPS1 (192.168.10.70) |  |  |  |

What you should observe. The first four tests succeed (devices in the same LAN, switched at Layer 2). The test to the gateway and the test to another LAN fail (typically "Request timed out" or "Destination host unreachable"), because the router interfaces are shut down. Notice that the first ping of a new destination often shows one lost reply. That happens because the PC first has to learn the MAC address with ARP (you will study this in Part G).

CHECKPOINT: Checkpoint C: ipconfig /all screenshots from at least four devices, MAC address table filled, test table completed with explanations. Save as v2.

# 12. PART D: Configure the Switches

A new Cisco switch works as soon as it is powered on, but it is insecure and anonymous: it has no password, no name and no management address. In this part you perform the basic switch configuration that every technician must be able to do from memory.

## 12.1 Concepts you must understand first

### Cisco IOS command modes

| Mode | Prompt | How to enter | What you can do |
| --- | --- | --- | --- |
| User EXEC | Switch> | Default after login | Basic monitoring commands (limited) |
| Privileged EXEC | Switch# | enable | All show commands, copy, reload, ping |
| Global configuration | Switch(config)# | configure terminal | Settings that affect the whole device: hostname, passwords, banner |
| Interface configuration | Switch(config-if)# | interface fastEthernet 0/1 | Settings for one interface or a range |
| Line configuration | Switch(config-line)# | line console 0 or line vty 0 15 | Console and remote-login settings |

Useful keys: Tab completes a command, ? shows help, Ctrl+C or end returns to privileged mode, exit goes back one level, and the up arrow repeats the previous command. A command can be shortened as long as it is unambiguous (conf t = configure terminal). Put no in front of a command to undo it.

### Management address on a switch

A switch forwards frames at Layer 2, so its ports have no IP address. To manage it remotely it needs an IP address on a virtual interface (SVI). By default all ports belong to VLAN 1, so the address is placed on interface vlan 1. For the switch to answer a technician on another LAN, it also needs a default gateway (ip default-gateway).

TASK D1  Open the CLI of SW-ADMIN

Click SW-ADMIN and open the CLI tab. If you see "Would you like to enter the initial configuration dialog?" type no and press Enter. Optionally, connect a PC to the switch console port with a console cable and use the Terminal application (9600 baud) to practise the way a real technician connects.

TASK D2  Basic configuration of SW-ADMIN

Type each block, read the comments (lines beginning with !), and check the prompt after each command. The comments explain why you type each line.

SW-ADMIN: identity and access security

Switch> enable

Switch# configure terminal

! 1. Give the device a name (shows in the prompt and in logs)

Switch(config)# hostname SW-ADMIN

! 2. Stop the switch treating mistyped words as host names to look up (avoids long delays)

SW-ADMIN(config)# no ip domain-lookup

! 3. Protect privileged mode with an ENCRYPTED (hashed) secret

SW-ADMIN(config)# enable secret Ucu@Enable2026

! 4. Protect the console line

SW-ADMIN(config)# line console 0

SW-ADMIN(config-line)# password Ucu@Con2026

SW-ADMIN(config-line)# login

SW-ADMIN(config-line)# logging synchronous

SW-ADMIN(config-line)# exec-timeout 5 0

SW-ADMIN(config-line)# exit

! 5. Protect remote (Telnet) logins on the 16 virtual terminal lines

SW-ADMIN(config)# line vty 0 15

SW-ADMIN(config-line)# password Ucu@Vty2026

SW-ADMIN(config-line)# login

SW-ADMIN(config-line)# exec-timeout 5 0

SW-ADMIN(config-line)# exit

! 6. Hide plain-text passwords in the configuration file

SW-ADMIN(config)# service password-encryption

! 7. Legal warning banner (the # is the delimiter that starts and ends the text)

SW-ADMIN(config)# banner motd # AUTHORISED UCU STAFF ONLY. ACTIVITY IS MONITORED. #

SW-ADMIN: management address, descriptions and saving

! 8. Management address on VLAN 1 and the default gateway for remote management

SW-ADMIN(config)# interface vlan 1

SW-ADMIN(config-if)# description Management SVI

SW-ADMIN(config-if)# ip address 192.168.10.2 255.255.255.192

SW-ADMIN(config-if)# no shutdown

SW-ADMIN(config-if)# exit

SW-ADMIN(config)# ip default-gateway 192.168.10.1

! 9. Describe each port you use (documentation inside the device)

SW-ADMIN(config)# interface fastEthernet 0/1

SW-ADMIN(config-if)# description PC-ADM1

SW-ADMIN(config-if)# interface fastEthernet 0/2

SW-ADMIN(config-if)# description PC-ADM2

SW-ADMIN(config-if)# interface fastEthernet 0/3

SW-ADMIN(config-if)# description Printer-ADM

SW-ADMIN(config-if)# interface gigabitEthernet 0/1

SW-ADMIN(config-if)# description Uplink to R-OFFICE G0/0

SW-ADMIN(config-if)# end

! 10. SAVE: running-config (RAM) is lost at power off; startup-config (NVRAM) is kept

SW-ADMIN# copy running-config startup-config

Destination filename [startup-config]?   (press Enter)

WARNING: Passwords are case sensitive, and the characters you type for a password are not displayed. The enable secret is stored hashed. The password lines are only weakly obscured by service password-encryption (type 7), which is why the strong protection is the enable secret and, later, SSH with local users. If a command is rejected with "% Invalid input detected", check: the mode (prompt), spelling, and the position of the ^ marker that points to the error.

TASK D3  Configure SW-OPS and SW-IT

Repeat the whole of D2 on the other two switches. Only the values in the table below change. The router engineer should check the commands of the switch engineer (peer review).

| Setting | SW-OPS | SW-IT |
| --- | --- | --- |
| hostname | SW-OPS | SW-IT |
| interface vlan 1 address | ip address 192.168.10.66 255.255.255.224 | ip address 192.168.10.98 255.255.255.240 |
| ip default-gateway | 192.168.10.65 | 192.168.10.97 |
| Fa0/1 description | PC-OPS1 | Server-IT |
| Fa0/2 description | PC-OPS2 | PC-IT1 |
| Fa0/3 description | Laptop-OPS | PC-IT2 |
| Gi0/1 description | Uplink to R-OFFICE G0/1 | Uplink to R-OFFICE G0/2 |

TASK D4  Verify the switch configuration

Run each command on each switch. For every command, write in your report what the output tells you (do not only paste it).

Verification commands

SW-ADMIN# show running-config

SW-ADMIN# show startup-config

SW-ADMIN# show ip interface brief

SW-ADMIN# show interfaces status

SW-ADMIN# show interfaces vlan 1

SW-ADMIN# show vlan brief

SW-ADMIN# show version

SW-ADMIN# show flash:

SW-ADMIN# show history

| Command | What to look for and explain |
| --- | --- |
| show running-config | Hostname, secret (hashed with $), encrypted line passwords, banner, SVI address. Which lines would you change to change the console password? |
| show ip interface brief | Vlan1 has the management address, status up/up. Which physical ports are up and which are down? |
| show interfaces status | Each port: description, connected or notconnect, VLAN 1, duplex and speed (auto, a-100 and so on) |
| show vlan brief | All ports start in VLAN 1 (default). What does this mean for the broadcast domain of the switch? |
| show version | IOS version, uptime, model, number of interfaces, configuration register |

TASK D5  Observe how a switch learns MAC addresses

A switch builds its MAC address table by reading the source MAC address of every frame and recording the port on which it arrived. It forwards a frame by looking up the destination MAC. Unknown destinations are flooded out of all other ports.

- On SW-ADMIN type clear mac address-table dynamic (it may be clear mac-address-table dynamic in some versions), then show mac address-table. The table should be empty or almost empty.

- On PC-ADM1 run ping 192.168.10.11 (PC-ADM2).

- Run show mac address-table again. You should see the MAC addresses of PC-ADM1 and PC-ADM2 with ports Fa0/1 and Fa0/2 and type DYNAMIC.

- Compare the MAC addresses with the ones you recorded in Part C. They must be identical.

- Ping the printer (192.168.10.20) and repeat. A third entry appears on Fa0/3.

Example output (your MAC addresses will differ)

SW-ADMIN# show mac address-table

Mac Address Table

-------------------------------------------

Vlan    Mac Address       Type        Ports

----    -----------       --------    -----

1    0001.4321.1111    DYNAMIC     Fa0/1       <- PC-ADM1 (example only)

1    0002.1642.2222    DYNAMIC     Fa0/2       <- PC-ADM2 (example only)

Questions: (1) Why does the router MAC address not appear yet? (2) How many collision domains and how many broadcast domains does SW-ADMIN create? (3) What does a switch do when it receives a frame whose destination MAC is not in its table?

CHECKPOINT: Checkpoint D: configuration of the three switches saved; show running-config and show ip interface brief screenshots; MAC address table evidence; answers to D5 questions. Save as v3.

# 13. PART E: Configure the Router

The router joins the three LANs. It has one interface in each LAN, and each interface address is the default gateway of that LAN. Because the router is directly connected to all three networks, it automatically knows how to reach them (connected routes). No routing protocol is needed in this lab.

## 13.1 Concepts

- A router forwards packets (Layer 3) using its routing table. For each packet it finds the route that best matches the destination IP address, then it re-frames the packet for the next link.

- A router changes the Layer 2 header at every hop (new source and destination MAC), but the source and destination IP addresses stay the same. The TTL field decreases by 1 at every router.

- Router interfaces are administratively down by default. You must give them an IP address and activate them with no shutdown.

- Every interface of a router is a separate broadcast domain. Broadcasts do not cross the router.

TASK E1  Basic configuration of R-OFFICE

R-OFFICE: identity and access security

Router> enable

Router# configure terminal

Router(config)# hostname R-OFFICE

R-OFFICE(config)# no ip domain-lookup

R-OFFICE(config)# enable secret Ucu@Enable2026

R-OFFICE(config)# service password-encryption

R-OFFICE(config)# banner motd # AUTHORISED UCU STAFF ONLY. ACTIVITY IS MONITORED. #

!

R-OFFICE(config)# line console 0

R-OFFICE(config-line)# password Ucu@Con2026

R-OFFICE(config-line)# login

R-OFFICE(config-line)# logging synchronous

R-OFFICE(config-line)# exec-timeout 5 0

R-OFFICE(config-line)# exit

R-OFFICE(config)# line vty 0 4

R-OFFICE(config-line)# password Ucu@Vty2026

R-OFFICE(config-line)# login

R-OFFICE(config-line)# exec-timeout 5 0

R-OFFICE(config-line)# exit

R-OFFICE: interfaces (gateways of the three LANs)

! Each interface = default gateway of one LAN. Use the table in Section 8.3.

R-OFFICE(config)# interface gigabitEthernet 0/0

R-OFFICE(config-if)# description LAN 1 Admin - to SW-ADMIN Gi0/1

R-OFFICE(config-if)# ip address 192.168.10.1 255.255.255.192

R-OFFICE(config-if)# no shutdown

R-OFFICE(config-if)# exit

R-OFFICE(config)# interface gigabitEthernet 0/1

R-OFFICE(config-if)# description LAN 2 Operations - to SW-OPS Gi0/1

R-OFFICE(config-if)# ip address 192.168.10.65 255.255.255.224

R-OFFICE(config-if)# no shutdown

R-OFFICE(config-if)# exit

R-OFFICE(config)# interface gigabitEthernet 0/2

R-OFFICE(config-if)# description LAN 3 IT - to SW-IT Gi0/1

R-OFFICE(config-if)# ip address 192.168.10.97 255.255.255.240

R-OFFICE(config-if)# no shutdown

R-OFFICE(config-if)# end

R-OFFICE# copy running-config startup-config

After no shutdown you will see messages such as "%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up" and "%LINEPROTO-5-UPDOWN ... changed state to up". Wait about 30 seconds for the switch ports to turn green.

TASK E2  Verify the router configuration

Verification commands

R-OFFICE# show ip interface brief

R-OFFICE# show interfaces gigabitEthernet 0/0

R-OFFICE# show ip route

R-OFFICE# show running-config

R-OFFICE# show arp

Expected ~show ip interface brief~: the three Gigabit interfaces have the correct addresses and show Status up and Protocol up. If Status is administratively down, you forgot no shutdown. If Status is up but Protocol is down, or Status is down, check the cable and the switch end.

Expected routing table

R-OFFICE# show ip route

Codes: C - connected, L - local, S - static, O - OSPF ...

Gateway of last resort is not set

192.168.10.0/24 is variably subnetted, 6 subnets, 3 masks

C        192.168.10.0/26 is directly connected, GigabitEthernet0/0

L        192.168.10.1/32 is directly connected, GigabitEthernet0/0

C        192.168.10.64/27 is directly connected, GigabitEthernet0/1

L        192.168.10.65/32 is directly connected, GigabitEthernet0/1

C        192.168.10.96/28 is directly connected, GigabitEthernet0/2

L        192.168.10.97/32 is directly connected, GigabitEthernet0/2

Explain in your report: the meaning of the codes C (connected network) and L (local address of the router itself, a /32 route); why the table says "variably subnetted, 6 subnets, 3 masks"; why there is no gateway of last resort (there is no connection to the Internet in this lab); and how the router would find the route for a packet addressed to 192.168.10.100 (it matches 192.168.10.96/28, the longest prefix).

TASK E3  Test connectivity between the LANs

From the Command Prompt of each PC run the following tests. Remember: the first ping may lose one packet because of ARP.

On PC-ADM1

C:\> ping 192.168.10.1          (own gateway)

C:\> ping 192.168.10.65         (gateway of LAN 2)

C:\> ping 192.168.10.97         (gateway of LAN 3)

C:\> ping 192.168.10.100        (Server-IT in LAN 3)

C:\> tracert 192.168.10.100

Expected tracert output (PC-ADM1 to Server-IT)

C:\> tracert 192.168.10.100

Tracing route to 192.168.10.100 over a maximum of 30 hops:

1   0 ms   0 ms   0 ms   192.168.10.1

2   0 ms   0 ms   0 ms   192.168.10.100

Trace complete.

Interpret the output: hop 1 is the default gateway (R-OFFICE G0/0). Hop 2 is the destination. The TTL value in successful ping replies is typically 128 inside the same LAN and 127 after crossing the router, because the router decreased the TTL by 1.

All nine end devices must now be able to ping each other and the three router interfaces. Create a full test matrix in your report. If one test fails, do not guess: use the troubleshooting method in Section 18.

CHECKPOINT: Checkpoint E: R-OFFICE configuration saved; show ip interface brief, show ip route and tracert screenshots; ping matrix with no failures. Save as v4.

# 14. PART F: IPv6 Addressing (Dual Stack)

IPv4 has only about 4.3 billion addresses, and they have run out. IPv6 uses 128-bit addresses and gives every device a globally unique address. During the long transition, networks run both protocols at the same time, which is called dual stack. In this part you add IPv6 beside the IPv4 you have already configured. Nothing you did before is removed.

## 14.1 Concepts

| Topic | What you must know |
| --- | --- |
| Format | 128 bits written as eight groups of four hexadecimal digits separated by colons, for example 2001:0DB8:ACAD:0010:0000:0000:0000:0001 |
| Rule 1: leading zeros | Leading zeros in a group may be dropped: 0DB8 becomes DB8, 0010 becomes 10, 0000 becomes 0 |
| Rule 2: double colon | One run of consecutive all-zero groups may be replaced by :: (only once per address): 2001:DB8:ACAD:10::1 |
| Prefix and interface ID | A /64 is normal for a LAN: the first 64 bits are the network prefix (here 2001:DB8:ACAD:10), the last 64 bits identify the interface |
| Global unicast | Routable on the Internet (2000::/3). The lab uses the documentation prefix 2001:DB8::/32, which is reserved for examples |
| Link-local | FE80::/10. Valid only on one link; every IPv6 interface has one; used for neighbor discovery and often as the gateway address |
| Multicast | FF00::/8. FF02::1 = all nodes on the link, FF02::2 = all routers on the link. IPv6 has no broadcast |
| Neighbor Discovery (NDP) | Replaces ARP. Uses ICMPv6 Neighbor Solicitation and Neighbor Advertisement to map an IPv6 address to a MAC address |

TASK F1  Abbreviation exercise (by hand)

| Full address | Shortest correct form |
| --- | --- |
| 2001:0DB8:ACAD:0020:0000:0000:0000:0001 |  |
| 2001:0DB8:ACAD:0030:0000:0000:0000:0100 |  |
| FE80:0000:0000:0000:0000:0000:0000:0001 |  |
| 2001:0DB8:0000:0000:ABCD:0000:0000:0001 |  |

Answers to check: 2001:DB8:ACAD:20::1, 2001:DB8:ACAD:30::100, FE80::1 and 2001:DB8::ABCD:0:0:1 (the last address has two equal runs of zeros; the usual convention compresses the first one, and 2001:DB8:0:0:ABCD::1 is also valid). Explain why the last address cannot use :: twice.

TASK F2  Enable IPv6 on R-OFFICE

R-OFFICE: IPv6 (dual stack with the existing IPv4 configuration)

R-OFFICE# configure terminal

! Without this command the router does NOT forward IPv6 packets

R-OFFICE(config)# ipv6 unicast-routing

R-OFFICE(config)# interface gigabitEthernet 0/0

R-OFFICE(config-if)# ipv6 address 2001:db8:acad:10::1/64

R-OFFICE(config-if)# ipv6 address fe80::1 link-local

R-OFFICE(config-if)# exit

R-OFFICE(config)# interface gigabitEthernet 0/1

R-OFFICE(config-if)# ipv6 address 2001:db8:acad:20::1/64

R-OFFICE(config-if)# ipv6 address fe80::1 link-local

R-OFFICE(config-if)# exit

R-OFFICE(config)# interface gigabitEthernet 0/2

R-OFFICE(config-if)# ipv6 address 2001:db8:acad:30::1/64

R-OFFICE(config-if)# ipv6 address fe80::1 link-local

R-OFFICE(config-if)# end

R-OFFICE# copy running-config startup-config

NOTE: Using the same link-local address FE80::1 on all three interfaces is allowed, because a link-local address only has to be unique on its own link. It also makes the gateway easy to remember.

TASK F3  Configure IPv6 on the end devices

- On each PC and laptop open Desktop > IP Configuration. In the IPv6 Configuration section choose Static.

- Enter the IPv6 Address with prefix length 64 from Section 8.4 (for PC-ADM1: 2001:DB8:ACAD:10::10 / 64) and the IPv6 Gateway (2001:DB8:ACAD:10::1).

- Do the same for PC-ADM2, PC-OPS1, PC-OPS2, Laptop-OPS, Server-IT (Server-IT: Config > FastEthernet0 > IPv6 Configuration, static), PC-IT1 and PC-IT2.

- Leave the IPv4 settings unchanged. The devices now have two addresses and a link-local address that Packet Tracer creates automatically.

TASK F4  Verify and test IPv6

On the router

R-OFFICE# show ipv6 interface brief

R-OFFICE# show ipv6 route

R-OFFICE# show ipv6 neighbors

On PC-ADM1

C:\> ipconfig

C:\> ping 2001:db8:acad:10::1

C:\> ping 2001:db8:acad:20::70

C:\> ping 2001:db8:acad:30::100

C:\> tracert 2001:db8:acad:30::100

Expected result: show ipv6 interface brief lists each interface with its link-local address (FE80::1) and its global address. show ipv6 route shows C (connected) and L (local) routes for the three /64 prefixes. All pings succeed. After the pings, show ipv6 neighbors lists the IPv6-to-MAC mappings learned by NDP, which is the IPv6 equivalent of the ARP table.

Write in your report: (1) the four differences between IPv4 and IPv6 that you saw in this lab (address length, notation, no broadcast, NDP instead of ARP, link-local addresses); (2) why dual stack is used in networks that are moving to IPv6; (3) what happens if you forget ipv6 unicast-routing (test it: remove it with no ipv6 unicast-routing and ping across LANs, then restore it).

CHECKPOINT: Checkpoint F: abbreviation answers, show ipv6 interface brief, successful IPv6 pings between all three LANs, written answers. Save as v5.

# 15. PART G: Services and Protocol Analysis

Networks exist to carry applications. In this part you add two services to Server-IT (HTTP and DNS), use them from other LANs, and then use Packet Tracer Simulation mode to watch how the protocols you study in CCNA 1 behave: ARP, ICMP, DNS (UDP), HTTP (TCP), and how the OSI and TCP/IP models describe each step.

## 15.1 Concepts

| OSI layer | TCP/IP layer | PDU name | Examples in this lab | Addresses used |
| --- | --- | --- | --- | --- |
| 7 Application | Application | Data | HTTP, DNS | Names and URLs |
| 6 Presentation | Application | Data | Text and image formats (HTML) | n/a |
| 5 Session | Application | Data | Management of the HTTP session | n/a |
| 4 Transport | Transport | Segment (TCP) / Datagram (UDP) | TCP port 80 (HTTP), UDP port 53 (DNS) | Port numbers |
| 3 Network | Internet | Packet | IPv4, IPv6, ICMP, routing | IP addresses |
| 2 Data link | Network access | Frame | Ethernet, ARP, switching | MAC addresses |
| 1 Physical | Network access | Bits | UTP cable, link lights | Signals |

| Protocol | Layer | Job | Key facts |
| --- | --- | --- | --- |
| ARP | 2 / 3 | Finds the MAC address that belongs to a known IPv4 address on the same network | Broadcast request, unicast reply; results kept in the ARP cache; hosts ARP for the gateway when the destination is remote |
| ICMP | 3 | Reports errors and tests reachability (ping, tracert) | Echo request (type 8), echo reply (type 0), TTL exceeded, destination unreachable |
| TCP | 4 | Reliable, connection-oriented delivery | Three-way handshake (SYN, SYN-ACK, ACK), sequence and acknowledgement numbers, flow control, FIN to close |
| UDP | 4 | Fast, connectionless delivery with no guarantee | No handshake; used by DNS queries, DHCP, voice |
| DNS | 7 | Translates names to IP addresses | Query to UDP port 53; record types A (IPv4), AAAA (IPv6) |
| HTTP | 7 | Transfers web pages | Client request (GET) and server response (200 OK) over TCP port 80 |

TASK G1  Configure DNS and HTTP on Server-IT

- Click Server-IT, open the Services tab and choose HTTP. Make sure HTTP is On. Click index.html (edit) and replace the content with the text below. Click Save.

- Choose DNS in the Services list. Set the DNS Service to On.

- Add two A records: Name intranet.office.local with Address 192.168.10.100, click Add; then Name printer.office.local with Address 192.168.10.20, click Add.

- Check that every end device has DNS server = 192.168.10.100 (Part C).

index.html on Server-IT (HTTP service)

<html>

<body>

<h1>UCU Kampala Small Office Intranet</h1>

<p>Welcome. This page is served by Server-IT (192.168.10.100).</p>

<p>Built by CCNA 1 Group X: (write your group member names here)</p>

</body>

</html>

TASK G2  Use the services from other LANs

- On PC-ADM1 open Desktop > Web Browser. Enter http://192.168.10.100 and press Go. The page appears (access by IP address).

- Enter http://intranet.office.local. The same page appears (access by name, DNS works).

- Open Command Prompt and run nslookup intranet.office.local and ping intranet.office.local. Note the IP address returned.

- Repeat the browser test from PC-OPS1 and PC-IT1. All three LANs must reach the page.

- Try ping printer.office.local from PC-OPS1.

Troubleshooting hint: if the name does not resolve but the IP address works, check the DNS server value on that PC, that DNS is On, and the spelling of the record. If neither works, return to the connectivity tests in Part E.

TASK G3  Watch ARP and ICMP in Simulation mode

- Click Simulation (bottom right of the Packet Tracer window). Click Edit Filters, click Show All/None to clear everything, then tick only ARP and ICMP (and later DNS, HTTP, TCP, UDP).

- On PC-ADM1 open the Command Prompt and clear its ARP cache: arp -d. Check it is empty with arp -a.

- In the Command Prompt type ping 192.168.10.100 and press Enter. In Simulation mode Packet Tracer places the packets into the Event List and waits. Click Capture / Forward repeatedly to move one step at a time.

- For every event click the coloured envelope on the diagram or the event in the list. Open the Inbound PDU Details and Outbound PDU Details tabs and record the fields in the table below.

The correct sequence of events for the first ping from PC-ADM1 to Server-IT is:

| Step | Event | Who sends it | Layer 2 destination | What it proves |
| --- | --- | --- | --- | --- |
| 1 | ARP request: "Who has 192.168.10.1?" | PC-ADM1 | FFFF.FFFF.FFFF (broadcast) | The PC must reach its gateway first because the destination is remote |
| 2 | ARP reply | R-OFFICE G0/0 | PC-ADM1 MAC (unicast) | The PC learns the router MAC and stores it in the ARP cache |
| 3 | ICMP echo request | PC-ADM1 | R-OFFICE G0/0 MAC | Source IP = PC-ADM1, destination IP = Server-IT: the IP address is not the router address, but the frame is addressed to the router |
| 4 | ARP request: "Who has 192.168.10.100?" | R-OFFICE G0/2 | FFFF.FFFF.FFFF | The router needs the MAC of the server on LAN 3 |
| 5 | ARP reply | Server-IT | Router G0/2 MAC | Router learns the server MAC |
| 6 | ICMP echo request (forwarded) | R-OFFICE G0/2 | Server-IT MAC | New Layer 2 header; same IP addresses; TTL decreased by 1 |
| 7 | ICMP echo reply | Server-IT | R-OFFICE G0/2 MAC | Return path: reply goes to the gateway |
| 8 | ICMP echo reply (forwarded) | R-OFFICE G0/0 | PC-ADM1 MAC | Reply reaches the source |

Complete this table from your own simulation (use MAC addresses from ipconfig /all and show interfaces on the router):

| Segment | Source MAC | Destination MAC | Source IP | Destination IP | TTL |
| --- | --- | --- | --- | --- | --- |
| PC-ADM1 to R-OFFICE (LAN 1) |  |  |  |  |  |
| R-OFFICE to Server-IT (LAN 3) |  |  |  |  |  |

Key conclusion to write in your report: the IP addresses stay the same from source to destination, but the MAC addresses change at every router hop. Now run the ping a second time. Which events disappear, and why?

TASK G4  Watch DNS (UDP) and HTTP (TCP)

- In Simulation mode, edit filters to show DNS, UDP, TCP and HTTP.

- On PC-OPS1 open the Web Browser and enter http://intranet.office.local. Step through the events with Capture / Forward.

- Find the DNS query: which transport protocol and destination port does it use? (UDP 53). Find the DNS response and the address it contains.

- Find the TCP three-way handshake (SYN, SYN-ACK, ACK) between PC-OPS1 and port 80 of Server-IT. Open the PDU and record the flags, the source port (a random high number) and the destination port 80.

- Find the HTTP GET request and the HTTP 200 OK response carrying the web page. Find the TCP FIN that closes the connection.

| Observation | Your answer |
| --- | --- |
| Transport protocol and ports used by the DNS query |  |
| Source port chosen by the PC for HTTP; destination port |  |
| Number of events in the TCP handshake and what each one means |  |
| Why does DNS use UDP while HTTP uses TCP? |  |
| At which OSI layers does the PDU change from Data to Segment to Packet to Frame to Bits? (use the OSI Model tab) |  |

TASK G5  Encapsulation and the OSI Model tab

Click any PDU in Simulation mode, then the OSI Model tab. Packet Tracer shows what each layer does with the data at the sending device (Outbound, layers 7 to 1) and at the receiving device (Inbound, layers 1 to 7). Take screenshots for one HTTP request at the PC (outbound) and at the server (inbound) and write for each layer one sentence describing what it did (for example: "Layer 4 added TCP header with destination port 80"). This is encapsulation and de-encapsulation.

CHECKPOINT: Checkpoint G: web page by IP and by name from all three LANs, nslookup output, filled event tables, TCP handshake evidence, OSI Model screenshots. Switch back to Realtime mode. Save as v6.

# 16. PART H: Basic Network Security

A network that works but can be broken into is not finished. In this part you apply the basic security measures that every technician must know: encrypted remote access (SSH), port security and disabling unused ports.

## 16.1 Concepts

| Threat | Why it is dangerous | Countermeasure in this part |
| --- | --- | --- |
| Telnet remote login | Telnet sends the user name and password in clear text, so anyone sniffing the network can read them | Replace Telnet with SSH (encrypted) |
| Unauthorised device plugged into a switch port | An outsider can connect a laptop in a meeting room and join the network | Port security (limits and remembers MAC addresses) |
| Unused open ports | Free doors for attackers or accidents | Shut down unused ports |
| Weak or shared passwords | Easily guessed or leaked | Strong passwords, local user accounts, hashed secrets, banner and timeouts |

TASK H1  Enable SSH on the switches and the router

SSH needs a host name, a domain name, an RSA key pair, a local user and VTY lines that accept only SSH. Perform this on SW-ADMIN first, then on the other devices.

SW-ADMIN (repeat on SW-OPS, SW-IT and R-OFFICE with their own host names)

SW-ADMIN# configure terminal

SW-ADMIN(config)# ip domain-name office.local

SW-ADMIN(config)# username netadmin privilege 15 secret Ucu@Admin2026

! The key size is the number of bits. At least 768 bits is required for SSH version 2.

SW-ADMIN(config)# crypto key generate rsa modulus 2048

SW-ADMIN(config)# ip ssh version 2

SW-ADMIN(config)# ip ssh time-out 60

SW-ADMIN(config)# ip ssh authentication-retries 3

SW-ADMIN(config)# line vty 0 15

SW-ADMIN(config-line)# transport input ssh

SW-ADMIN(config-line)# login local

SW-ADMIN(config-line)# exit

SW-ADMIN(config)# end

SW-ADMIN# copy running-config startup-config

PACKET TRACER: Set the hostname before generating the key, because the key name is built from the host name and the domain name. If you see "Please define a domain-name first", the ip domain-name command is missing. On R-OFFICE use line vty 0 4 instead of line vty 0 15. After transport input ssh the old VTY password is no longer used, because login local checks the local user database.

Test SSH. From the Command Prompt of PC-ADM1:

On PC-ADM1

C:\> ssh -l netadmin 192.168.10.2         (SW-ADMIN, same LAN)

Password: Ucu@Admin2026

SW-ADMIN#

C:\> ssh -l netadmin 192.168.10.66        (SW-OPS from another LAN, via the router)

C:\> ssh -l netadmin 192.168.10.1         (the router)

C:\> telnet 192.168.10.2                  (must now FAIL: connection refused)

Verify with show ip ssh and show users (shows who is connected). Explain in your report: why remote management of the switch in another LAN works only because you configured ip default-gateway on the switch.

TASK H2  Port security on SW-ADMIN

Port security limits the number of MAC addresses allowed on a port and decides what happens when a violation occurs. The sticky option makes the switch learn the first MAC addresses and add them to the running configuration.

SW-ADMIN: port security on the three user ports

SW-ADMIN# configure terminal

SW-ADMIN(config)# interface range fastEthernet 0/1 - 3

SW-ADMIN(config-if-range)# switchport mode access

SW-ADMIN(config-if-range)# switchport port-security

SW-ADMIN(config-if-range)# switchport port-security maximum 1

SW-ADMIN(config-if-range)# switchport port-security mac-address sticky

SW-ADMIN(config-if-range)# switchport port-security violation shutdown

SW-ADMIN(config-if-range)# end

SW-ADMIN# show port-security

SW-ADMIN# show port-security interface fastEthernet 0/1

SW-ADMIN# show port-security address

- Generate traffic so that the switch learns the MAC addresses: ping from PC-ADM1 to the gateway and to PC-ADM2. Run show port-security address. The MAC addresses appear as SecureSticky.

- Violation test. Swap the cables of PC-ADM1 and PC-ADM2 (PC-ADM1 to Fa0/2 and PC-ADM2 to Fa0/1) and ping the gateway. The switch sees a MAC address that is not allowed. The ports change to err-disabled (the link lights turn red). Confirm with show interfaces status and show port-security interface fastEthernet 0/1 (look at Port Status: Secure-shutdown and Security Violation Count).

- Recovery. Put the cables back. On each shut-down port type shutdown and then no shutdown in interface configuration mode. The port becomes active again.

- Explain the three violation modes: protect (drops frames silently), restrict (drops frames and logs/counts), shutdown (the port is err-disabled). Why is shutdown the default and the strictest?

TASK H3  Shut down every unused port

SW-ADMIN (repeat on SW-OPS and SW-IT; also apply port security to their Fa0/1 - 3)

SW-ADMIN# configure terminal

SW-ADMIN(config)# interface range fastEthernet 0/4 - 24

SW-ADMIN(config-if-range)# description UNUSED

SW-ADMIN(config-if-range)# shutdown

SW-ADMIN(config-if-range)# exit

SW-ADMIN(config)# interface gigabitEthernet 0/2

SW-ADMIN(config-if)# description UNUSED

SW-ADMIN(config-if)# shutdown

SW-ADMIN(config-if)# end

SW-ADMIN# copy running-config startup-config

SW-ADMIN# show interfaces status

Check that all unused ports show disabled in show interfaces status and that Fa0/1 - 3 and Gi0/1 are still connected. Take care not to shut down the uplink Gi0/1.

TASK H4  Security review

Complete a security checklist for every network device and add it to your report. A cell may be filled only with evidence (a command output).

| Device | Hashed enable secret | Console password | Banner | SSH only (no Telnet) | Port security | Unused ports shut | Config saved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R-OFFICE |  |  |  |  | n/a | n/a |  |
| SW-ADMIN |  |  |  |  |  |  |  |
| SW-OPS |  |  |  |  |  |  |  |
| SW-IT |  |  |  |  |  |  |  |

CHECKPOINT: Checkpoint H: SSH login evidence from a PC in another LAN, Telnet refused, port-security violation and recovery evidence, show interfaces status for each switch, security checklist. Save as v7.

# 17. PART I: Testing and Verification

A network is finished only when you can prove that it works, and that it fails where it should. Run the complete test plan below and record the result of each test with evidence. Repeat the plan after every important change.

## 17.1 How to record evidence

- Each screenshot must show the device name, the command and the full output.

- For each test, write: ID, purpose, steps, expected result, actual result, PASS or FAIL.

- Run tests from end devices, as a user would, not only from router consoles.

- If a test fails, do not change anything until you have diagnosed it (Section 18).

## 17.2 Functional test plan

| ID | Test | Expected result | Part |
| --- | --- | --- | --- |
| T01 | All end devices: ipconfig /all | Address, mask, gateway and DNS match Section 8.3 | C |
| T02 | PC-ADM1 to PC-ADM2 (same LAN) | Success; TTL 128 | C |
| T03 | PC-ADM1 to gateway 192.168.10.1 | Success | E |
| T04 | PC-ADM1 to PC-OPS1 (another LAN) | Success; TTL 127; first ping may lose one packet | E |
| T05 | PC-OPS1 to Server-IT | Success | E |
| T06 | PC-IT2 to Printer-ADM | Success | E |
| T07 | tracert 192.168.10.100 from PC-ADM1 | Two hops: 192.168.10.1 then 192.168.10.100 | E |
| T08 | IPv6 ping PC-ADM1 to Server-IT (2001:db8:acad:30::100) | Success | F |
| T09 | nslookup intranet.office.local from PC-OPS1 | Returns 192.168.10.100 | G |
| T10 | Browser on PC-ADM2 to http://intranet.office.local | Intranet page opens | G |
| T11 | ARP: arp -a on PC-ADM1 after T04 | Entry for 192.168.10.1 with the router MAC (not the MAC of PC-OPS1) | G |
| T12 | SSH from PC-ADM1 to SW-OPS (192.168.10.66) | Banner, login succeeds | H |
| T13 | Telnet to any device | Fails (refused) | H |
| T14 | Swap the cables of PC-ADM1 and PC-ADM2 (port security) | Ports err-disabled; recovered with shutdown / no shutdown | H |
| T15 | show mac address-table on each switch after traffic | MAC addresses of connected devices on the correct ports | D |
| T16 | Reload R-OFFICE and SW-ADMIN, then repeat T03 and T12 | Still works: the configuration was saved to NVRAM | D, E |

TIP: Test 16 proves you saved the configuration. In Packet Tracer use reload in privileged mode and answer confirm. If your network is broken after the reload, you forgot copy running-config startup-config on that device.

CHECKPOINT: Checkpoint I: completed test table with evidence. Any failed test must be followed by a fix and a successful re-test.

# 18. Troubleshooting Challenge

Troubleshooting is the most valuable skill of a network technician. In this challenge your group must introduce at least three faults into your own network (or into the copy of another group), diagnose them, fix them, and prove the fix. The lecturer may add one more fault during your presentation.

## 18.1 A method that works

- Define the problem exactly. Who is affected? Which service? Since when? "The network is down" is not a problem statement. "PC-OPS1 cannot ping Server-IT, but PC-ADM1 can" is.

- Gather facts with ipconfig, ping, tracert and show commands. Change nothing yet.

- Choose a method. Bottom-up: check physical (cables, lights), then data link (switch ports), then network (IP, mask, gateway), then application. Divide and conquer: ping the gateway first, then the next hop, then the destination. Compare with a device that works.

- Make one hypothesis and test it. Change one thing at a time.

- Verify from the user side, then re-run the test plan to make sure that nothing else broke.

- Document the ticket (template below).

| Where the ping stops | Likely cause | What to check |
| --- | --- | --- |
| Cannot ping its own IP address | Network card or address not configured | ipconfig; is the NIC enabled? Is a static address set? |
| Can ping own LAN, cannot ping the gateway | Wrong gateway, mask, or router interface down | Compare the host mask and gateway with Section 8.3; show ip interface brief on the router |
| Can ping the gateway, cannot ping another LAN | Router problem, destination problem or wrong mask on the destination | show ip route; ping the destination from the router; check the destination configuration |
| Can ping by IP, cannot ping by name | DNS problem | DNS server address on the host; DNS service and records on the server |
| Link light is red | Physical problem | Cable type, correct port, interface shutdown, device power |

## 18.2 Fault catalogue (choose at least three, from different Parts)

| Fault to introduce | Symptom | Diagnosis | Correct fix |
| --- | --- | --- | --- |
| Wrong subnet mask on PC-OPS1 (255.255.255.192 instead of .224) | Some remote hosts unreachable; behaviour depends on the destination address | ipconfig; apply the AND operation by hand | Set 255.255.255.224 |
| Wrong default gateway on PC-IT1 (192.168.10.65) | PC-IT1 reaches its own LAN but nothing else | ipconfig; ping the gateway fails | Set 192.168.10.97 |
| Router interface shut down (G0/1) | Entire LAN 2 isolated; red link light at the router | show ip interface brief shows administratively down | no shutdown on G0/1 |
| Wrong cable type (crossover between SW-IT and PC-IT2) | Link stays down | Link light; replace and compare | Use a straight-through cable |
| Typing error in the server IP (192.168.10.110 instead of .100) | Ping by the correct address fails; names resolve to the old address | ping; show ip route; ipconfig on the server | Correct the address, verify the DNS record |
| DNS server missing on Laptop-OPS | Ping by IP works; browsing by name fails | nslookup | Add 192.168.10.100 as DNS |
| Switch port in the wrong place or port shutdown (Fa0/2) | One device offline | show interfaces status | no shutdown or recable |
| Missing ~ip default-gateway~ on SW-OPS | Switch answers inside LAN 2 but SSH from other LANs fails | show running-config | Add ip default-gateway 192.168.10.65 |
| Port security violation | A user cannot connect after a device change | show port-security interface | Remove the unauthorised device, then shutdown / no shutdown |

How to run the challenge. Group A closes its eyes (or leaves the room) while Group B introduces one fault in Group A's file. Group A then follows the method above, and Group B watches and scores the method. Swap roles afterwards.

## 18.3 Trouble ticket template (copy one per fault)

| Field | Entry |
| --- | --- |
| Ticket number, date, diagnosing member | TT-01 / ____ / (name) |
| Problem statement (user view) |  |
| Fault introduced (hidden from the diagnoser) |  |
| Diagnosis: commands used and what each output showed |  |
| Cause (root cause, not just the symptom) |  |
| Solution: exact commands or actions |  |
| Verification: test that proves it is fixed |  |
| Lesson learned / how to prevent it |  |

# 19. Command Reference

You must be able to use these commands and explain their output. Do not just type them. In the presentation the lecturer may ask what a command shows and why you chose it.

## 19.1 On PCs, laptops and servers (Command Prompt)

| Command | What it shows or does |
| --- | --- |
| ipconfig and ipconfig /all | IPv4 and IPv6 addresses, mask, gateway; with /all also MAC address and DNS server |
| ping address | ICMP echo test. Shows reply time and TTL. Request timed out = no reply; Destination host unreachable = a router or the host reports that it cannot deliver |
| tracert address | The path (each router hop) to the destination, using ICMP with increasing TTL |
| arp -a and arp -d | Display and clear the ARP cache (IP address to MAC address) |
| nslookup name | Asks the DNS server for the address of a name |
| ssh -l user address | Opens an SSH session to a device |

## 19.2 On Cisco switches and routers (IOS)

| Area | Command | What it tells you |
| --- | --- | --- |
| Modes | enable, configure terminal, exit, end | Move between User EXEC, Privileged EXEC and configuration modes |
| Identity | hostname, banner motd, no ip domain-lookup | Name the device, legal warning, avoid DNS lookup delay |
| Passwords | enable secret, line console 0, line vty, service password-encryption, login local | Protect privileged mode, console, remote access; hide plain text |
| Interfaces | interface, description, ip address, ipv6 address, no shutdown, shutdown | Configure and activate interfaces |
| Management | interface vlan 1, ip default-gateway | Switch management address and gateway |
| Save | copy running-config startup-config | Keeps the configuration after a reload |
| Verify config | show running-config, show startup-config | Active configuration (RAM) and saved configuration (NVRAM) |
| Verify interfaces | show ip interface brief, show interfaces, show interfaces status, show ipv6 interface brief | Status and protocol of each interface; addresses; errors; speed and duplex |
| Verify switching | show mac address-table, show vlan brief | Learned MAC addresses and ports; ports in VLAN 1 |
| Verify routing | show ip route, show ipv6 route, show arp, show ipv6 neighbors | Routing table (C and L codes); IP-to-MAC mappings |
| Verify security | show ip ssh, show users, show port-security, show port-security interface | SSH version and sessions; port security state and violations |
| Device facts | show version, show flash:, show clock, show history | IOS image, uptime, memory, commands you have typed |
| Reset | erase startup-config then reload | Returns a device to factory configuration (Appendix C) |

# 20. Required Network Documentation

A technician who builds a network without documenting it leaves a problem for the next technician. Prepare all the items below. They must agree exactly with your Packet Tracer file.

## 20.1 Network topology diagram

Redraw Figure 1 yourselves (in Packet Tracer, with a drawing tool or on paper then scanned). It must clearly show:

- All devices with their host names and device types (router, switch, PC, laptop, printer, server).

- The interface numbers at both ends of every link.

- The IPv4 address and prefix of every router interface and the network address of every LAN.

- The IPv6 prefix of every LAN.

- A title, group number, date and a legend (cable types).

## 20.2 IP addressing table

Use the table in Section 8.3, extended with the IPv6 addresses of Section 8.4 and the MAC address of each device (from ipconfig /all). Add the router and switch interface MAC addresses (from show interfaces).

## 20.3 Subnetting table

The completed table from Task A2, with all calculations shown (binary conversion, host bits, mask, network, first, last, broadcast, usable hosts) and a note on the unused address space.

## 20.4 Switch port table

| Switch | Port | Connected device | Description in IOS | Mode / security | Status |
| --- | --- | --- | --- | --- | --- |
| SW-ADMIN | Fa0/1 | PC-ADM1 | PC-ADM1 | access, port security (sticky, max 1) | up |
| (complete for all used ports of the three switches, the Gi0/1 uplinks, and one line for the shut-down unused ports) |  |  |  |  |  |

## 20.5 Other documents

| Document | Content |
| --- | --- |
| Configuration files | One text file per device (R-OFFICE.txt, SW-ADMIN.txt, SW-OPS.txt, SW-IT.txt) copied from show running-config after the last save. Passwords may stay as in the lab |
| Test plan | Section 17 table with results and screenshots |
| Troubleshooting tickets | At least three completed tickets (Section 18.3) |
| Security checklist | Task H4 table with evidence |
| Protocol analysis notes | Part G event tables, handshake evidence, OSI screenshots |

# 21. Group Discussion Questions

Discuss these questions before and during the lab. Write short, clear answers in your report. Every member must be able to answer every question.

### Network fundamentals and addressing

- What is the difference between a LAN and a WAN? Which is your office network?

- What is the difference between a hub, a switch and a router? At which OSI layer does each work?

- Why do computers need both a MAC address and an IP address? Which one changes at each router?

- What is the purpose of a subnet mask? What does a computer do with it before sending a packet?

- Why did we use VLSM instead of equal-sized subnets? How many addresses would have been wasted with three /26 subnets?

- What is the default gateway? What happens when it is configured wrongly?

- Why are the addresses 192.168.10.0 and 192.168.10.63 not usable as host addresses in LAN 1?

- Why are private addresses (192.168.0.0/16, 10.0.0.0/8, 172.16.0.0/12) used inside offices? What would be needed to reach the Internet?

### Switching, routing and protocols

- How does a switch learn MAC addresses, and what does it do with an unknown destination?

- How many collision domains and how many broadcast domains exist in your network? Explain with the diagram.

- Why do broadcasts not cross the router?

- Why does the first ping often fail, and the following pings succeed?

- What does the TTL field do? What would happen to a packet in a routing loop if there were no TTL?

- What is the difference between TCP and UDP? Which one do HTTP and DNS use, and why?

- What is the difference between the OSI model and the TCP/IP model? Map the layers to the protocols in your lab.

- What does the routing table of R-OFFICE contain, and what do the codes C and L mean?

### Security and management

- Why is SSH preferred to Telnet?

- What is the difference between enable password and enable secret? Between running-config and startup-config?

- How does port security protect an office? What are the three violation modes?

- Why should unused switch ports be shut down?

- Give three reasons why documentation is as important as configuration.

- What is IPv6 dual stack, and why is it used during the transition?

# 22. The Packet Journey Challenge

At the presentation, every group must explain, using its own devices and addresses, the complete journey of the following request. The lecturer will choose the source and the destination.

Scenario. A user at PC-OPS1 (192.168.10.70) opens the browser and enters http://intranet.office.local. The page is held on Server-IT (192.168.10.100). Explain what happens from the moment the user presses Enter until the page is displayed.

| Stage | What you must explain |
| --- | --- |
| 1. Application | The browser needs an IP address for the name. It asks the DNS client. HTTP will be used afterwards |
| 2. DNS query | UDP datagram, destination port 53, to the DNS server 192.168.10.100. Is it local or remote? Perform the AND operation with the mask /27 to decide |
| 3. ARP for the gateway | PC-OPS1 has no MAC address for the gateway 192.168.10.65. It sends an ARP broadcast. The router answers. The MAC is cached |
| 4. Frame to the router | Source MAC = PC-OPS1, destination MAC = R-OFFICE G0/1. Source IP = 192.168.10.70, destination IP = 192.168.10.100. The switch looks in its MAC table and forwards the frame out of the uplink |
| 5. Routing | R-OFFICE removes the Ethernet header, reads the destination IP, searches the routing table (match 192.168.10.96/28 via G0/2), decreases TTL, builds a new frame |
| 6. ARP on LAN 3 | R-OFFICE needs the MAC of Server-IT. It sends an ARP broadcast out of G0/2 and learns the MAC |
| 7. Delivery | New frame: source MAC = R-OFFICE G0/2, destination MAC = Server-IT. The IP addresses did not change |
| 8. DNS reply | The server returns the A record 192.168.10.100 in a UDP datagram. The reply follows the reverse path |
| 9. TCP handshake | SYN, SYN-ACK, ACK from a random source port to destination port 80, through the same path |
| 10. HTTP | GET request, 200 OK response with the page, TCP acknowledgements, then FIN to close |
| 11. Physical | Every frame is converted to bits and sent as electrical signals over the UTP cable |

In your explanation use these words correctly: encapsulation, PDU (data, segment, packet, frame, bits), MAC address, IP address, port number, ARP, default gateway, routing table, TTL, broadcast, unicast, switch MAC table, DNS, TCP, UDP, HTTP.

# 23. Presentation Structure

Each group presents and demonstrates live in Packet Tracer (about 25 minutes) and then answers questions (about 10 minutes). The network, not the slides, is the evidence. Every member speaks and the lecturer chooses who answers each question.

| Part | Content | Time |
| --- | --- | --- |
| 1 | Scenario and requirements; what the office needs | 2 min |
| 2 | Design: topology, addressing and VLSM decisions (show the subnet table) | 3 min |
| 3 | Build: cabling, link lights, end-device configuration | 3 min |
| 4 | Switches and router: show the saved configuration and the key verification commands | 4 min |
| 5 | Inter-LAN connectivity and IPv6: live pings and tracert | 3 min |
| 6 | Services and protocols: DNS and web by name; show one ARP and one TCP handshake in Simulation mode | 4 min |
| 7 | Security: SSH, port security violation and recovery, unused ports | 3 min |
| 8 | Troubleshooting: fix an introduced fault using Problem, Diagnosis, Solution, Verification | 3 min |
| 9 | Questions and the Packet Journey Challenge | 10 min |

# 24. Required Deliverables

Submit one folder (zip) per group named UCU_CCNA1_GroupX with these items:

| No. | Deliverable | Details |
| --- | --- | --- |
| 1 | Packet Tracer file | CCNA1_GroupX_final.pkt, fully configured, saved (copy running-config startup-config on every device) |
| 2 | Topology diagram | CCNA1_GroupX_Topology.png or PDF; contains everything listed in Section 20.1 |
| 3 | Technical report | CCNA1_GroupX_Report.docx or PDF (structure below) |
| 4 | Configuration files | Four text files, one per network device |
| 5 | Evidence folder | Screenshots named by test ID (T01, T02 ...) and by Part (A1, B3 ...) |
| 6 | Individual contribution statement | Half a page per member: what you did, what you learned, signed |

## 24.1 Technical report structure

| Chapter | Content |
| --- | --- |
| 1. Cover page and group members | Group number, names, student numbers, roles |
| 2. Introduction | Purpose of the network and summary of what was built |
| 3. Scenario and requirements | In your own words |
| 4. Design | Topology diagram, device list, cable list, justification of design |
| 5. IP addressing plan | Subnetting calculations (Part A), IPv4 and IPv6 tables |
| 6. Configuration | Switch configuration, router configuration, end devices, with an explanation of key commands |
| 7. Verification and testing | Test plan and evidence |
| 8. Protocol analysis | ARP, ICMP, DNS, TCP, HTTP, OSI mapping (Part G) |
| 9. Security | Part H checklist and explanation |
| 10. Troubleshooting | At least three tickets |
| 11. Challenges and solutions | What went wrong and how you solved it |
| 12. Conclusion | What you learned; what could be improved |
| 13. References | Course notes, Cisco Networking Academy, books and web pages, with a consistent style |
| Appendices | Full device configurations |

# 25. Assessment Rubric

| Assessment area | What earns full marks | Marks |
| --- | --- | --- |
| Part A: planning and subnetting | Correct binary conversions and VLSM table; clear working; written explanations; labelled sketch | 15 |
| Part B: physical build | Correct devices, names and cable types; all links green; port labels visible | 10 |
| Part C: end devices | Correct IPv4 settings on all devices; ipconfig evidence; correct predictions | 10 |
| Part D: switches | Complete baseline configuration on three switches; saved; MAC table analysis; verification explained | 15 |
| Part E: router | Correct interfaces, descriptions, passwords and banner; routing table explained; full ping matrix | 15 |
| Part F: IPv6 | Dual stack working; abbreviation exercise correct; NDP explained | 5 |
| Part G: services and protocols | DNS and HTTP working; ARP, ICMP, TCP, UDP observed and explained; OSI mapping | 10 |
| Part H: security | SSH only, port security demonstrated, unused ports disabled, checklist | 5 |
| Part I and Section 18: testing, troubleshooting and documentation | Complete test plan; three documented faults; accurate diagram and tables | 5 |
| Presentation and individual participation | Clear live demonstration; every member explains and answers about the whole network | 10 |
| TOTAL |  | 100 |

NOTE: Marks are awarded for understanding as well as for working configurations. A working network that a member cannot explain loses marks. Copied work, or configurations that do not match the report, will lose marks.

# 26. Individual Participation

Although this is a group lab, individual understanding is assessed. The lecturer may choose any member at random and ask them to do any of the following, live:

- Explain the topology and trace a packet from any device to any other, including the MAC and IP addresses on each link.

- Calculate a subnet: the network, broadcast, first and last usable addresses and the number of usable hosts, for any given address and prefix.

- Decide, by the AND operation, whether two addresses are in the same network.

- Configure a switch or router interface, a password, SSH or port security from memory.

- Interpret show ip interface brief, show ip route, show mac address-table, ipconfig /all, arp -a or tracert output.

- Explain what a given ping result means and troubleshoot a fault introduced by the lecturer.

- Explain a TCP three-way handshake, the difference between TCP and UDP, and the layers of the OSI and TCP/IP models.

WARNING: Every student is responsible for understanding the complete network, not only the part they personally configured. Rotate the keyboard so that every member configures at least one switch, the router and several end devices.

# 27. Extension Challenges

Finished early, and everything is verified? Attempt these optional challenges for extra learning (they are not required, and they do not replace any part above). Document each one in a separate section of your report.

TASK X1  Automatic addressing with DHCP and a relay agent

Idea. The administrator wants LAN 2 users to receive their addresses automatically. A DHCP server on Server-IT is in another LAN, and DHCP discovery is a broadcast that stops at the router. The router interface needs a relay agent (ip helper-address) that forwards the broadcast to the server as a unicast.

- On Server-IT open Services > DHCP, set the service On and add a pool: Pool name OPS-POOL, Default gateway 192.168.10.65, DNS server 192.168.10.100, Start IP 192.168.10.80, Subnet mask 255.255.255.224, Maximum users 10. Click Add then Save.

- On R-OFFICE configure the relay (commands below).

- On a new laptop or PC placed in LAN 2 choose DHCP in IP Configuration. Check that it receives an address from 192.168.10.80 to 192.168.10.89 and the correct gateway and DNS.

- Explain the four DHCP messages (Discover, Offer, Request, Acknowledgement) and use Simulation mode to capture them.

R-OFFICE: DHCP relay for LAN 2

R-OFFICE(config)# interface gigabitEthernet 0/1

R-OFFICE(config-if)# ip helper-address 192.168.10.100

R-OFFICE(config-if)# end

R-OFFICE# copy running-config startup-config

TASK X2  Plan the growth of the office

The office will add a Reception area (12 devices) and a Store (6 devices). Using the free space from 192.168.10.112 onwards, calculate the subnets (VLSM, largest first), write the new rows for the addressing table, and explain what hardware you need (your router has only three Gigabit interfaces) and which later CCNA topics solve this problem (VLANs and router-on-a-stick).

TASK X3  Wireless access

Add a wireless access point (AccessPoint-PT) to SW-OPS and connect a wireless laptop or smartphone. Set an SSID, WPA2 password and verify that the wireless client receives an address in LAN 2. List the security settings you chose and why.

# 28. Expected Outcome

At the end of this lab your group has a working, secured, tested and documented small office network, and each member can explain why it works. You have practised the CCNA 1 path from planning to presenting:

From concept to explanation

UNDERSTANDING -> PLANNING -> BUILDING -> CONFIGURING -> VERIFYING

-> ANALYSING PROTOCOLS -> SECURING -> TROUBLESHOOTING -> DOCUMENTING -> EXPLAINING

The goal is not only a Packet Tracer file that works. The goal is that you understand how addressing, switching, routing and protocols cooperate to move data across a network, and that you can defend your design to a lecturer today and to an employer tomorrow. These skills are the foundation for CCNA 2 (VLANs, routing, security) and CCNA 3 (enterprise networking, management and automation).

# Appendix A: Reference Configurations

Use these configurations to compare with your own after you have finished. Do not copy and paste them before you have attempted the configuration yourselves. The lecturer may ask you to explain any line.

R-OFFICE (final state after Parts E, F and H; crypto key generated separately)

hostname R-OFFICE

no ip domain-lookup

ip domain-name office.local

enable secret Ucu@Enable2026

service password-encryption

username netadmin privilege 15 secret Ucu@Admin2026

banner motd # AUTHORISED UCU STAFF ONLY. ACTIVITY IS MONITORED. #

ipv6 unicast-routing

!

interface GigabitEthernet0/0

description LAN 1 Admin - to SW-ADMIN Gi0/1

ip address 192.168.10.1 255.255.255.192

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:10::1/64

no shutdown

interface GigabitEthernet0/1

description LAN 2 Operations - to SW-OPS Gi0/1

ip address 192.168.10.65 255.255.255.224

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:20::1/64

no shutdown

interface GigabitEthernet0/2

description LAN 3 IT - to SW-IT Gi0/1

ip address 192.168.10.97 255.255.255.240

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:30::1/64

no shutdown

!

ip ssh version 2

line con 0

password Ucu@Con2026

login

logging synchronous

exec-timeout 5 0

line vty 0 4

transport input ssh

login local

exec-timeout 5 0

end

SW-ADMIN (final state after Parts D and H; SW-OPS and SW-IT differ only in names, addresses and descriptions)

hostname SW-ADMIN

no ip domain-lookup

ip domain-name office.local

enable secret Ucu@Enable2026

service password-encryption

username netadmin privilege 15 secret Ucu@Admin2026

banner motd # AUTHORISED UCU STAFF ONLY. ACTIVITY IS MONITORED. #

ip default-gateway 192.168.10.1

ip ssh version 2

!

interface FastEthernet0/1

description PC-ADM1

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

interface FastEthernet0/2

description PC-ADM2

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

interface FastEthernet0/3

description Printer-ADM

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation shutdown

interface range FastEthernet0/4 - 24

description UNUSED

shutdown

interface GigabitEthernet0/1

description Uplink to R-OFFICE G0/0

interface GigabitEthernet0/2

description UNUSED

shutdown

interface Vlan1

description Management SVI

ip address 192.168.10.2 255.255.255.192

no shutdown

!

line con 0

password Ucu@Con2026

login

logging synchronous

exec-timeout 5 0

line vty 0 15

transport input ssh

login local

exec-timeout 5 0

end

# Appendix B: Group Submission Checklist

| Check | Done |
| --- | --- |
| Packet Tracer file saved; startup-config equals running-config on all four network devices | [ ] |
| All device names, addresses and ports agree with Section 8 | [ ] |
| Subnet worksheet complete and correct, with working shown | [ ] |
| Ping matrix shows success between every pair of end devices (IPv4) and across LANs (IPv6) | [ ] |
| DNS and web page work by name from all three LANs | [ ] |
| SSH works, Telnet is refused, port security tested, unused ports shut down | [ ] |
| Topology diagram and all tables completed | [ ] |
| At least three troubleshooting tickets completed | [ ] |
| Screenshots show device names and complete output | [ ] |
| Every member can explain the whole network | [ ] |

# Appendix C: Common Mistakes and How to Reset a Device

- Forgetting ~no shutdown~ on router interfaces. Link light stays red and show ip interface brief says "administratively down".

- Wrong mask on a host. Remember that the three LANs use three different masks (/26, /27, /28).

- Wrong gateway. Each LAN has its own gateway (.1, .65 and .97).

- Not saving. Run copy running-config startup-config after every important step.

- Using the automatic cable instead of choosing the correct cable type.

- Typing a command in the wrong mode. Check the prompt. If it says (config-if)# you are inside an interface.

- Setting the hostname after generating the SSH key. Set the hostname and domain name first.

- Shutting down the uplink Gi0/1 when shutting down unused ports.

- Testing too early. Wait for amber ports to turn green; use the fast-forward button.

- Testing only the first ping. The first ping may lose a reply because of ARP; send at least four.

Reset a switch to factory settings

Switch# erase startup-config      (confirm)

Switch# delete vlan.dat           (switches only; confirm)

Switch# reload                    (answer "no" if asked to save the configuration)

Reset a router to factory settings

Router# erase startup-config      (confirm)

Router# reload                    (answer "no" if asked to save the configuration)